Basler Standard Light Flat Dome-200x200-x#

Basler Standard Flat Dome Lights can be used in various applications for bright field lighting of slightly objects.

Key Features#

- Internal LED controller with 2 operating modes

- Compatible with Basler SLP Strobe Controller

- Preconfigured hardware components

- Various sizes and LED colors

General Specifications#

|  |  | Basler Standard Light Flat Dome-200x200-White |
| --- | --- | --- |
| Order Number |  | 2200000732 |
| LED Color |  | White |
| Luminous Area |  | 200 x 200 mm |
| Lens Hole (diameter) |  | 45 mm |
| Recommended Light Working Distance |  | 100–500 mm |
| Conformity |

LED Specifications#

|  | Flat Dome Light Series 200x200-x |
| --- | --- |
| Luminous Power of White LEDs (lumen) | 2090 |
| Radiant Power of Red LEDs (mW) | 6150 |
| Radiant Power of IR LEDs (mW) | 2960 |

Electrical Specifications#

|  | Flat Dome Light Series 200x200-x |
| --- | --- |
| Required Power for Voltage Control Mode | 24 VDC ± 10 %, 22 W |
| Required Current for Current Control Mode | Red, White, and Blue: 1200 mA Infrared: 1500 mA |
| Recommended max. LED Current for Strobe Mode (50 % Duty Cycle, 100 ms Pulses) | Red, White, and Blue: 2400 mA Infrared: Must never be used. |
| Recommended max. LED Current for Strobe Mode (25 % Duty Cycle, 20 ms Pulses) | Red, White, and Blue: 3600 mA Infrared: Must never be used. |
| Recommended max. LED Current for Strobe Mode (10 % Duty Cycle, 5 ms Pulses) | Red, White, and Blue: 4800 mA Infrared: 3000 mA |
| Infrared only: Recommended max. LED Current for Strobe Mode (1 % Duty Cycle, 100 µs Pulses) | 4500 mA |
| Recommended min. Flash Time in Strobe Mode | 100 µs |

Mechanical Specifications#

|  | Flat Dome Light Series 200x200-x |
| --- | --- |
| Dimensions (H x W x D) | 240 x 230 x 12 mm |
| Weight | 1150 g |
| Material | Black anodized aluminum housing with PMMA light cover |
| Connector | M8 socket, 4-pin male connector |

Please contact OEM Automatic for technical assistance.

Email: vision@oem.co.uk

Telephone: 01162849900